# Virtual Vahana 2026 – Phase 1 Technical Report

Project: CARLA ADAS Fusion System

Team Name: NextVolt

Team Members: Karan Rajesh, Vaikunth MS, Tharun Kumaar RK, Mahesh Paul J, Sagar Sewak, Francis Solomon X, Harishankar MN

## 1. Problem Statement

Urban and highway driving environments demand highly integrated Advanced Driver-Assistance Systems (ADAS) that operate without mode conflicts. Standard simulator implementations often lack production-grade sensor fusion, resulting in "ghost braking" on slopes or erratic steering corrections at high speeds. The objective of this project is to develop a robust, Level 2 assisted driving suite within the CARLA simulator that prioritizes safety over convenience. This requires engineering a deterministic arbitration layer that seamlessly manages Lane Keeping Assist (LKA), Adaptive Cruise Control (ACC), and Autonomous Emergency Braking (AEB) while maintaining absolute driver-in-the-loop awareness through a clear Human-Machine Interface (HMI).

## 2. System Overview

The ADAS Fusion System is developed entirely in Python, utilizing the CARLA Python API to interface with a simulated vehicle physics engine running at a locked 60 Hz.

- Simulator Framework: CARLA 0.9.11

- Technology Stack: Python 3.12, PyGame (for HMI rendering), NumPy (for matrix and threshold calculations).

- Key Software Classes: World, ADAS (managing ACC and FCW), LKAController, PIDController, RadarSensor, SemanticLidarSensor, and custom HUD rendering classes (including a Tesla-style spatial dashboard).

- Control Paradigm: Keyboard-controllable manual driving overlaid with modular, suppressible ADAS assists.

## 3. Methodology

Our engineering approach follows a strict Perception → Fusion → Decision → Actuation pipeline.

- Perception & Fusion: Raw sensor data from Radar, Cameras, and Raycast sensors are inherently noisy. We process this data through a custom "Ghost Filter," cross-referencing kinematic distances with Semantic LiDAR and IMU pitch data to eliminate false positives caused by road geometry (e.g., steep slopes).

- Decision & Arbitration: We established a strict hierarchical state machine. Active safety is paramount; therefore, control priority strictly follows: AEB > ACC > LKA > Manual Input.

- HMI Design: To prevent driver distraction, the UI is split between a classic heads-up display and a consolidated right-side telemetry panel, supplementing situational awareness with a top-down spatial mini-map.

## 4. Algorithms & Calculations

### 4.1 Autonomous Emergency Braking (AEB) & Forward Collision Warning (FCW)

The AEB system calculates risk using both dynamic Time-To-Collision (TTC) and static kinematic stopping distances (inspired by UNECE R152 and ISO 22737 standards).

Time-To-Collision:

TTC=distancemax0,-vrel⋅direction

- TTC ≤ 1.2 s → State 3 (Full Emergency Brake, ~1g deceleration)

- TTC ≤ 1.8 s → State 2 (Partial Brake, 10% to 45% pressure)

- TTC ≤ 2.6 s → State 1 (Visual/Audio Warning)

Kinematic Thresholds (based on Ego Speed vmps and reaction time treact=1.2s):

dwarn=vmps⋅1.2+0.5⋅0.35⋅9.8⋅1.22+6.0

dpartial=vmps⋅1.2+vmps22⋅0.35⋅9.8+4.0

dfull=vmps⋅0.8+vmps22⋅9.81+2.5

Persistence & Hold: A threat must persist for 3 consecutive frames before triggering State ≥ 2. If the vehicle is stopped and an obstacle is detected at < 3.2m, a "Standstill Hold" latches the brakes until the driver engages reverse.

### 4.2 Adaptive Cruise Control (ACC)

Longitudinal speed control utilizes a PID controller with tuned constants: Kp=1.0, Ki=0.05, Kd=0.1.

Safe Following Distance:

dsafe=vkmh3.6⋅time_gap+6.0

(Default time_gap = 2.0s)

Target Velocity Logic:

- No Lead Vehicle: Maintain driver-set cruise speed.

- Lead Detected: Target velocity dynamically adjusts to ego speed plus 2.0⋅lead_dist-dsafe.

- Red Light (No lead): Target velocity smoothly drops to 0.0.

### 4.3 Lane Keeping Assist (LKA)

The LKA utilizes a MathWorks 4-Stage architecture. To prevent "snap" overcorrections at highway speeds, three stability algorithms are applied per frame:

- Speed-Adaptive Authority: Authority is 100% (0.50 multiplier) at ≤ 60 km/h, tapering linearly to 56% (0.28 multiplier) at 130 km/h.

- Rate Limiter: Caps maximum steering delta: max_change=0.22vmps

- IIR Smoothing Filter:

yn=α⋅xn+1-α⋅yn-1

(α shifts from 0.55 in city driving to 0.25 on highways ≥ 100 km/h).

Departure Thresholds: Triggers at lateral_dev>0.45m and clears atlateral_dev<0.22m.

## 5. System Architecture

The system relies on a continuous data loop executing at 60 Hz.

Data Flow Overview:

- Sensors: RGB/Depth/Semantic Cameras, Front Radar (35° FOV), Rear Radar, Semantic LiDAR (64 ch, 50m), Obstacle Raycast, IMU, GNSS.

- Fusion & Decision Layer: Combines Ground-truth smart threat detection with noisy sensor data to feed the AEB state machine, LKA controller, and ACC PID.

- Actuators & Output: Prioritized vehicle control commands (Throttle, Brake, Steer) are dispatched to the CARLA vehicle, while HMI data is sent to the PyGame UI.

## 6. Results & Observations

Extensive testing within the CARLA environment yielded the following observations:

- AEB Reliability: The system successfully brings the vehicle to a complete halt < 2m from static objects across a testing range of 0 to 130 km/h.

- Longitudinal Control: The ACC holds an exact 2.0s time gap and successfully arbitrates queue-following logic versus smooth red-light stopping.

- Lateral Stability: The LKA's IIR filter and rate limiter resulted in smooth, stable lane centering up to 130 km/h without hazardous oscillation.

- False Positive Reduction: The integration of the Semantic LiDAR and IMU pitch check successfully eliminated all observed false-positive AEB events caused by steep road inclines or curbs.

## 7. Challenges & Future Scope

Resolved Challenges:

- Radar Ghost Braking: Raw radar on inclines frequently triggered the AEB. Solved via a semantic masking and pitch-check filter.

- LKA High-Speed Instability: PID overcorrection at highway speeds was mitigated through our custom authority taper and IIR smoothing algorithms.

- Traffic Light Conflicts: Solved queue-follow logic by programming the ACC to yield to lead vehicles rather than braking abruptly for the light itself if a queue exists.

Future Enhancements (Phase 2):

- Transition from a PID controller to Model Predictive Control (MPC) for superior longitudinal smoothness.

- Integration of V2X (Vehicle-to-Everything) SPaT/MAP messaging for predictive intersection handling.

- Establishing a ROS2 bridge for potential real-world vehicle hardware testing.

## 8. Demo Video and Screenshots

Demo Video Gdrive Link

Fig 1: Dashboard HMI Screenshot

Fig 2: AEB Warning on Dashboard

Fig 3: AEB Emergency Breaking

Fig 4: ACC active at 30kmph

Fig 5: Lane Keeping Assist