# Feature Logic & Arbitration Explanation

Virtual Vahana 2026 – Phase 1 Submission

Team: NextVolt

Team Members: Karan Rajesh, Vaikunth MS, Tharun Kumaar RK, Mahesh Paul J, Sagar Sewak, Francis Solomon X, Harishankar MN

## 1. Overview

This document outlines the deterministic mathematical models and decision-making state machines governing our Advanced Driver-Assistance System (ADAS). Operating on a 60 Hz loop within the CARLA simulator, the system continuously calculates kinematics for Autonomous Emergency Braking (AEB), Adaptive Cruise Control (ACC), and Lane Keeping Assist (LKA).

## 2. Autonomous Emergency Braking (AEB) & Forward Collision Warning (FCW)

The AEB system is our primary active safety layer. It evaluates collision risks using a combination of dynamic Time-To-Collision (TTC) for moving targets and static kinematic stopping distances for stationary obstacles.

### 2.1 Time-To-Collision (TTC) Calculation

For dynamic actors (e.g., vehicles, pedestrians) detected via Radar and Semantic LiDAR, TTC is continuously evaluated:

TTC=DistancerelativeVelocityrelative

- Emergency Threshold: If TTC ≤1.2s, the system immediately triggers State 3 (100% brake demand).

- Partial Braking Threshold: If TTC ≤1.8s, the system triggers State 2.

- Warning Threshold: If TTC ≤2.6s, the system triggers State 1 (Visual/Audio alert).

### 2.2 Kinematic Safety Thresholds (Distance-Based)

To reliably handle stationary objects and sudden cut-ins, we calculate three dynamic distance thresholds based on the ego vehicle's current velocity (v) in meters per second, utilizing an assumed UNECE R152 driver reaction time (treact=1.2s).

- Warning Threshold (dwarn):

dwarn=v⋅treact+0.5⋅apartial⋅treact2+6.0

Action: Triggers State 1. Throttle is manually capped at 30% to prevent further acceleration.

- Partial Braking Threshold (dpartial):

dpartial=v⋅treact+v22⋅apartial+4.0

Action: Triggers State 2. To avoid jerky deceleration, brake pressure ramps linearly from 10% to 45% based on the vehicle's penetration depth into this zone.

- Emergency Braking Threshold (dfull):

dfull=v⋅0.8+v22⋅afull+2.5

Action: Triggers State 3. Demands 1.0 (100%) brake and cuts throttle to 0.0. LKA steering is zeroed out to return lateral control to the driver.

### 2.3 Sensor Fusion "Ghost Filter" & Persistence

- Ghost Filter: Hard braking pitches the vehicle down, causing the Raycast sensor to hit the asphalt. If Raycast detects an obstacle but the Semantic LiDAR classifies the points as "road surface" (tag = 7), the threat is ignored as a ghost.

- Persistence Latch: A threat must persist for 3 consecutive frames to trigger AEB State 2 or 3. If stopped and a wall is < 3.2m away, a "Standstill Hold" latches the brakes until reverse gear is engaged.

## 3. Adaptive Cruise Control (ACC) Logic

The ACC system regulates longitudinal speed to maintain a dynamic time gap, governed by a Proportional-Integral-Derivative (PID) controller.

### 3.1 Gap Maintenance & PID Tuning

The target safe distance scales with speed to maintain a 2.0-second time gap:

Distancesafe=vkmh3.6⋅2.0+6.0

The PID controller utilizes the following constants: Kp=1.0, Ki=0.05, Kd=0.1.

Output=Kp⋅et+Ki⋅etdt+Kd⋅detdt

Anti-Windup: To prevent integral windup during an AEB event, the PID's error sum is forcibly zeroed out when AEB demand > 0.0.

### 3.2 Traffic Sign & Light Logic

- Speed Limits: When a sign is detected within an 18-degree forward cone facing the ego vehicle, the ACC maximum target speed clamps to the recognized limit.

- Traffic Lights: If approaching a Red/Yellow light and the lead distance is > 15m (first in queue), the ACC target velocity smoothly drops to 0.0.

## 4. Lane Keeping Assist (LKA) Logic

Our LKA utilizes a 4-Stage architecture built on CARLA's waypoint tracking to maintain lateral stability.

- Departure Thresholds: A visual LDW alert triggers at 0.45m of lateral deviation. Hysteresis ensures the warning persists until the vehicle returns to within 0.22m of the lane center.

- Speed-Adaptive Authority: Steering assist is capped at 50% authority at speeds ≤60 km/h, scaling down linearly to 28% at 130 km/h.

- IIR Low-Pass Filter: To eliminate micro-jitter, the steering command passes through an Infinite Impulse Response (IIR) filter:

yn=α⋅xn+1-α⋅yn-1

Where α transitions from 0.55 (city driving) to 0.25 (highway driving).

## 5. Arbitration Logic & Feature Interaction

To prevent hazardous conflicting commands (e.g., ACC accelerating while AEB tries to stop), we engineered a strict state-machine hierarchy in the control loop.

- Level 1 (Absolute Priority): Autonomous Emergency Braking (AEB)

  - If AEB State ≥ 2, it physically overwrites the vehicle throttle to 0.0 and applies brake demand.

  - Lateral Arbitration: AEB sets the LKA steering contribution to 0.0. This intentionally disables LKA during an emergency stop so the driver can execute evasive maneuvers without fighting the steering wheel.

- Level 2 (Secondary Priority): Adaptive Cruise Control (ACC)

  - Only permitted to send throttle/brake commands if AEB demand is exactly 0.0.

  - Yields entirely to manual driver braking.

- Level 3 (Tertiary Priority): Manual Driver Input

  - If the driver actively steers (steer>0.05), LKA instantly releases control.

  - Manual braking stacks with AEB, but manual throttle is strictly ignored if AEB is active.